RFID Tag

|  | CybatiWorks™ MISSION: Simple Communications and Control |
| --- | --- |

|  | During this mission you will explore how to sense and control physical components and what adversaries can see if you do not protect your communications. The Internet of Things builds upon the decades of the Industrial Revolution and now can empower you with amazing cyber-physical powers. |
| --- | --- |

|  | Step 1. Physically place the RFID tag located on this instruction sheet on top of the black RFID reader located to the left of the workstation. Remove the instruction card after you hear the BEEP. Click OK to start the Mission wizard. The wizard will prepare your Mission and then open the Chromium web browser. If the browser states the site is unavailable, wait patiently as the screen will refresh. |
| --- | --- |
|  | Step 2. The wizard loaded a program that locally controls the light tree LEDs on your Printed Circuit Board (PCB) and sends a string “Hello World!” to one of the control stations LED boards on the left and/or right of the Main screen. In Node-RED, you can click the check-boxes next to each item on the left to send the commands. Also watch the Main screen as you send the LED board commands. Your messages are being intercepted wirelessly! The central station P3 Raspberry PI is equipped with a wireless antenna that is in monitor mode.. |
|  | Step 3. Double-click on the “Hello World” flow block and change the message and transmit it. You have completed this mission after you update the flow block and send your own message to the LED board. Remember to “Deploy” changes. |
|  | Step 4. This completes your mission. Tap the Reset card on to the RFID reader so that the next participant can begin their mission. |

|  | This mission simulates real world control systems that operate critical infrastructure such as traffic light intersections, the power grid, fluid and gas delivery systems. The mission also showed how easy it is to intercept and interpret unprotected wireless data using just a few lines of python code using a module called Scapy on the P2 station. You can continue to the next mission to learn more about cyber-physical systems, encryption and coding techniques. |
| --- | --- |

|  | This mission used a Raspberry PI as your computer, the control stations and as the Hacker. We encourage you to explore the cyber-physical world through the power of the Raspberry PI and the growing Maker community! |
| --- | --- |

RFID Tag

|  | CybatiWorks™ MISSION: Local Control with Boolean Logic |  |
| --- | --- | --- |
|  | This mission explores controlling local cyber-physical components using Boolean logic. Boolean logic includes the concepts of AND, OR, NAND, NOR binary analysis. |  |  |

|  | Step 1. Physically place the RFID tag located on this instruction sheet on top of the black RFID reader located to the left of the workstation. Remove the instruction card after you hear the BEEP. Click OK to start the Mission wizard. The wizard will prepare your Mission and then open the Chromium web browser. If the browser states the site is unavailable, wait patiently as the screen will refresh. |
| --- | --- |
|  | Step 2. Review the Boolean logic gate tables below. Boolean logic serves as the basic algorithm for cyber-physical control. The basic cyber-physical algorithm interprets sensor values to decide upon actuating outputs. The AND, OR constructs can be seen in the Node-RED functional flows. AND conditions require the inputs to be both TRUE for a 1 and OR conditions require at least 1 condition to be TRUE. AND. IN; IN; OUT. 0; 0; 0. 0; 1; 0. 1; 0; 0. 1; 1; 1.; OR. IN; IN; OUT. 0; 0; 0. 0; 1; 1. 1; 0; 1. 1; 1; 1.; NOT. IN; OUT. 0; 1. 1; 0.. |
|  | Step 3. The wizard loaded a program to explore local control with Boolean logic. You need to complete and deploy the flow by adding more flow connectors. Add a connector between the AND and PCB Red LED blocks by selecting the circle on the AND block and dragging the connection to the Red LED block. Complete the same connection for the Stop PCB button to the Invert (NOT) block. Click “Deploy” in the upper-right and try out your logic.. |
|  | Step 4. This completes your mission. Tap the Reset card on to the RFID reader so that the next participant can begin their mission. |

|  | Algorithms power many modern societies in the automated trading of stocks and goods as well as cyber-physical systems such as the electrical infrastructure, pipelines, airplanes, cranes, and automobiles. The algorithms have been designed and updated for decades as society has improved how to sense and control. |
| --- | --- |

|  | Continue to sharpen your logic design skills through the investigation of Node-RED capabilities and furthermore by reading up on PID loops. Proportional, Integral and Derivative. |
| --- | --- |

RFID Tag

|  | CybatiWorks™ MISSION: Remote Light Tree Control |  |
| --- | --- | --- |
|  | This mission explores using network communication to remotely control a cyber-physical system. The control stations each have a light tree located at the top. You will review and adjust the program flow controlling the light trees remotely. |  |  |

|  | Step 1. Physically place the RFID tag located on this instruction sheet on top of the black RFID reader located to the left of the workstation. Remove the instruction card after you hear the BEEP. Click OK to start the Mission wizard. The wizard will prepare your Mission and then open the Chromium web browser. If the browser states the site is unavailable, wait patiently as the screen will refresh. |
| --- | --- |
|  | Step 2. The wizard loaded a program that remotely controls the light trees located at each control station. The control stations are located at IP addresses in the 192.168.8.0/24 subnet. The LED lights in each tree are associated with specific Transmission Control Protocol (TCP) port numbers, TCP:10001, TCP:10002 and TCP:10003. These ports receive 0s and 1s to turn off and on the lights. The 2nd browser tab loaded is a Human Machine Interface (HMI) that has programmed light switches for each light. You can toggle the lights on and off with these switches. Note that two participant stations are associated with each light tree. Someone else may be toggling the lights as you are too.. |
|  | Step 3. Click the toggle switches on the HMI and checkboxes next to each block in the Node-RED flow-editor. Explore and adjust the logic so that one switch will turn on and off all LEDs. Also watch on the central station screen as your control messages are captured wirelessly by the Hacker. |
|  | Step 4. This completes your mission. Tap the Reset card on to the RFID reader so that the next participant can begin their mission. |

|  | Light trees are used in industrial automation all around the world. The light trees provide status information about the running process. Did you know that most light trees are programmed as RED is operational and GREEN is stopped? This is the complete opposite of a traffic light intersection. |
| --- | --- |

|  | Imagine what you could invent! You may become the next great inventor. There are also many great cyber-physical systems around the world that need to be improved and defended from cyberattacks. You may want to explore how they operate such as power grids, pipelines, manufacturing and self-driving automobiles. |
| --- | --- |

RFID Tag

|  | CybatiWorks™ MISSION: Protecting Information and Control |  |
| --- | --- | --- |
|  | This mission explores the technique of encryption to protect communications and control. Encryption helps to stop eavesdroppers from spying on data at rest, in transit and in process. You will enable the Advanced Encryption Standard (AES) to stop the Hacker from spying on your data while it is in transit wirelessly. |  |  |

|  | Step 1. Physically place the RFID tag located on this instruction sheet on top of the black RFID reader located to the left of the workstation. Remove the instruction card after you hear the BEEP. Click OK to start the Mission wizard. The wizard will prepare your Mission and then open the Chromium web browser. If the browser states the site is unavailable, wait patiently as the screen will refresh. |
| --- | --- |
|  | Step 2. The wizard loaded a program that remotely controls the control station LED board and the light tree Red LED. Click on the checkboxes with connected flows to send “Hello World” to the LED board and turn on and off the Red LED. The central hacking stations displayed on the main screen are also wirelessly collecting your command and control signaling as you send the data.. |
|  | Step 3. Add the flow connections to the appropriate encrypt block to the “Hello World” block and to the true and false blocks. Also add the flow connection between the encrypt blocks and the appropriate remote TCP connection. The TCP connections are associated with encrypt blocks on the control stations. Double-click on the encrypt blocks to configure the Algorithm type of AES and a Secret Key text of secretkey. Watch the central station Hacker screen as you turn on and off the light and send the message. The central station no longer is collecting clear text data, it is only collecting cipher text. |
|  | Step 4. This completes your mission. Tap the Reset card on to the RFID reader so that the next participant can begin their mission. |

|  | Encryption has been used for centuries. Julius Caesar’s cipher has been described as the Caesar cipher which is a ROT-3 (rotational 3, A = D, B = E, C = F, etc.) cipher. AES is more complex and can commonly be selected to operate in 3 modes, 128 bit, 192 bit and 256 bit. That is 256 unique 0s and 1s protecting your data! 000101010101010101010101010101010101010101010101011110000111100000101010101010101010101010101010101010101010101011110000111100000101010101010101010101010101010101010101010101011110000111100000101010101010101010101010101010101010101010101011110000111100 |
| --- | --- |

|  | Cryptosystems and signals intelligence bridge the gap between science and art. You can search more for phrases such as symmetric keys, asymmetric keys, Secure Hash Algorithm (SHA) hashing and steganography. |
| --- | --- |

RFID Tag

|  | CybatiWorks™ MISSION: Review the Control Station Flow Logic |  |
| --- | --- | --- |
|  | This mission explores how the control stations are programmed. You will review the logic and additional code added to the control blocks to support the remote control, local sensors and actuators. In another mission you will be tasked with exploring and making your own code so you can start getting ideas now! |  |  |

|  | Step 1. Physically place the RFID tag located on this instruction sheet on top of the black RFID reader located to the left of the workstation. Remove the instruction card after you hear the BEEP. Click OK to start the Mission wizard. The wizard will prepare your Mission and then open the Chromium web browser. If the browser states the site is unavailable, wait patiently as the screen will refresh. |
| --- | --- |
|  | Step 2. Now that you are getting used to the programming flows, let’s review the logic and HMI running on the control stations in more detail. Note, you will not be able to program a local Sense hat as your station does not have one directly attached. The Sense hat blocks will indicate “disconnected”. Double click on the different blocks to review the logic. The logic includes several TCP and Sense hat connections to support all of the missions. |
|  | Step 3. This completes your mission. Tap the Reset card on to the RFID reader so that the next participant can begin their mission. |

|  | Reviewing and disassembling code and electronics hardware is a common and beneficial practice, if performed ethically. If you are looking to invent and sell something, it is important to understand copyright and patent law to make sure you are not infringing on someone else’s rights. We should always give credit to the original inventors. |
| --- | --- |

|  | Node-RED is within the default installation of the Raspbian operating system for the Raspberry PI. Search for sensors, actuators and flows that allow you to learn how to defend today and invent the future! |
| --- | --- |

RFID Tag

|  | CybatiWorks™ MISSION: Central Station Hacker Tools |  |
| --- | --- | --- |
|  | This mission allows you to explore how the Hacker was catching your secrets using tools like Ettercap, EtherApe, scapy, python and specific ifconfig/iwconfig commands. The Hacker was monitoring the unprotected wireless communications of our network. |  |  |

|  | Step 1. Physically place the RFID tag located on this instruction sheet on top of the black RFID reader located to the left of the workstation. Remove the instruction card after you hear the BEEP. Click OK to start the Mission wizard. The wizard will prepare your Mission and then open the Chromium web browser. If the browser states the site is unavailable, wait patiently as the screen will refresh. |
| --- | --- |
|  | Step 2. The chromium browser opened four specific files for you to review. probemon.py : this command was run to monitor wireless SSID requests. Any 802.11 wireless device that is on sends probes for networks it remembers. This script collects and displays these probes. secrets.py : this script looks at specific TCP:10000 range ports and extracts the TCP payload from the packet. The TCP payload in this case on the ports we were monitoring included control and communications of the LEDs. rfid.py: this script is how we read the wizard ID cards and execute the program associated with the card ID. mitm_1.sh: this is the BASH script for Ettercap Commands to setup the Panda wireless USB - sudo ifconfig wlan1 down - sudo iwconfig wlan1 mode monitor - sudo ifconfig wlan1 up - sudo iw dev wlan1 set freq 2437. |
|  | Step 3. This completes your mission. Tap the Reset card on to the RFID reader so that the next participant can begin their mission. |

|  | Do you ever connect to unprotected wireless networks or store data unprotected? Have you ever looked to see what data is leaving your computer or phone and whether it is protected or not? Critical data and systems must be intentionally protected, else one must assume vulnerable and compromised. NOTE: Make sure you are aware of local laws before you monitor any communications that is not your own. |
| --- | --- |

|  | Wireless communications are everywhere. We just explored IEEE 802.11 wireless. You can search for IEEE 802.15, Bluetooth, vehicular Tire Pressure Monitoring System (TPMS) and many more. |
| --- | --- |

RFID Tag

|  | CybatiWorks™ MISSION: MAKE |  |
| --- | --- | --- |
|  | This mission allows you to explore some additional flows and make your own. After you have completed your work, please save your work locally on the participant station and notify us. |  |  |

|  | Step 1. Physically place the RFID tag located on this instruction sheet on top of the black RFID reader located to the left of the workstation. Remove the instruction card after you hear the BEEP. Click OK to start the Mission wizard. The wizard will prepare your Mission and then open the Chromium web browser. If the browser states the site is unavailable, wait patiently as the screen will refresh. |
| --- | --- |
|  | Step 2. Node-RED has launched with some sample programming flows. Inspect, make changes and have fun! Make sure to click “Deploy” to initiate your changes. |
|  | Step 3. If you are interested in saving your code, click on the ¦¦¦ in the upper right of the screen. Select Export and then Clipboard and then Download. The file will be saved in the /home/pi/Downloads/ directory. We can send this file to you if you are interested. |
|  | Step 4. This completes your mission. Tap the Reset card on to the RFID reader so that the next participant can begin their mission. |

|  | You can play an awesome role in inventing and defending the future way of life. Thank you for your participation in the CybatiWorks Missions. |
| --- | --- |

|  | Search online for maker movements and follow CYBATI and the Raspberry PI foundation. |
| --- | --- |

RFID Tag

|  | CybatiWorks™ MISSION: Traffic Light State Machine |  |
| --- | --- | --- |
|  | This mission explores the state machine of a simple U.S. traffic light intersection. The discrete states allow the traffic light to transition the outputs. |  |  |

|  | Step 1. Physically place the RFID tag located on this instruction sheet on top of the black RFID reader located to the left of the workstation. Remove the instruction card after you hear the BEEP. Click OK to start the Mission wizard. The wizard will prepare your Mission and then open the Chromium web browser. If the browser states the site is unavailable, wait patiently as the screen will refresh. |
| --- | --- |
|  | Step 2. The wizard has opened two unique windows, the traffic light state machine tables as a Libre Office document and two browser tabs with the flow logic and the Dashboard. The traffic light state transitions begin immediately upon the flow loading. Explore each flow block to see its configuration. Note the individual states listed in program are based on the road and color while the state tables in the document are based upon unique intersection states. The state transition timer table is written in Blockly, similar to Scratch programming. The Blockly programming node also allows you to easily convert the code to javascript. |
|  | Step 3. Think about how many inputs, outputs and timers are typically associated with a single traffic light intersection? The logic can become quite complex. Each intersection may also be influenced by pedestrian buttons, magnetic sensors, emergency vehicles, cameras, and other elements. The more items and data flows in the algorithm increases the challenge to secure the attack surface. Your mission is complete once you have successfully adjusted the timers to .25 seconds each. The simplest solution is to adjust the Blockly code. |
|  | Step 4. This completes your mission. Tap the Reset card on to the RFID reader so that the next participant can begin their mission. |

|  | Traffic light intersections are increasingly becoming points to locate additional inputs and outputs. Some cities now have environmental sensors, security cameras, data wireless sensors and are associating the inputs with artificial intelligence analysis tools. Each element and the associated database provide an attack surface that requires defense. |
| --- | --- |

|  | Research state machines and expand further in to the introduction of Proportional, Integral and Derivative (PID) loops. PID loops are state machine with integrated feedback based upon operational results. |
| --- | --- |

RFID Tag

|  | CybatiWorks™ MISSION: Remote Sensors with Wireshark |  |
| --- | --- | --- |
|  | This mission collects remote data from the control station and displays it on your local Dashboard. The remote data is temperature, humidity and barometric pressure collected from the Sense hat connected to the Raspberry PI control station. You will review the logic and then use Wireshark to review the captured network data. |  |  |

|  | Step 1. Physically place the RFID tag located on this instruction sheet on top of the black RFID reader located to the left of the workstation. Remove the instruction card after you hear the BEEP. Click OK to start the Mission wizard. The wizard will prepare your Mission and then open the Chromium web browser. If the browser states the site is unavailable, wait patiently as the screen will refresh. |
| --- | --- |
|  | Step 2. The simple flows shown by the wizard collects data from the TCP ports provided by the central stations. The temperature data is sent directly to a gauge and is also directed to a function to convert it to Celsius. The Dashboard tab provides the gauges for visual representations. |
|  | Step 3. The wizard also launched wireshark with a Display filter. Display filters limit what you see but do not limit what you capture. You can remove the filter to see how much data your eth0 is receiving. You can also re-apply the filter by selecting the drop-down arrow on the right of the filter section. Filters are paramount to find the needle in the haystack. In this case the haystack of collected data was not too deep, the data is payload of the TCP segment. Your mission is complete once you compare the values you are receiving with the values you are displaying in the Dashboard. |
|  | Step 4. This completes your mission. Tap the Reset card on to the RFID reader so that the next participant can begin their mission. |

|  | Remotely sensed data is part of a larger algorithm that makes decisions for both local and wide area operations. It is very important to confirm the correctness through integrity verification and sensor diversity. Manipulators may target the sensor directly, the communications channel to and from it, or both. |
| --- | --- |

|  | There is a great documentary called “Algorithms” available on NetFlix, check it out! |
| --- | --- |

RFID Tag

|  | CybatiWorks™ MISSION: Introduction to Physical Sensor Manipulation |  |
| --- | --- | --- |
|  | In this mission you will use a handheld laser to target and manipulate the Sense hat sensor readings for humidity and barometric pressure. The mission highlights the need for sensor diversity, physical protection, predictive learning, and value validation. |  |  |

|  | Step 1. Physically place the RFID tag located on this instruction sheet on top of the black RFID reader located to the left of the workstation. Remove the instruction card after you hear the BEEP. Click OK to start the Mission wizard. The wizard will prepare your Mission and then open the Chromium web browser. If the browser states the site is unavailable, wait patiently as the screen will refresh. Review the two documents provided in the CybatiWorks Search Engine. |
| --- | --- |
|  | Step 2. The loaded logic reads the values of the humidity, barometric pressure and temperature from the sensors on the control station Sense hat. These values are polled remotely by your workstation and displayed on the Node-RED Dashboard. The control station LED matrix will change its picture and the values will become erratic once you have properly targeted system. Ask a facilitator for a laser pointer. You must exercise caution to only point the active laser at the Sense hat PCB. Target a control station that is actively displaying the Dashboard gauges. You may need to explore the Sense hat to know where to target. |
|  | Step 3. The Sense hat values are polled every 1 second and displayed on the gauges. You will have to hold very still as you aim the laser at the correct location on the PCB located on station S1 or S3. Once you have caused an errant reading you can reset the LED matrix by selecting the checkbox next True in the logic. The S2 and S4 stations are actively polling at a very fast rate but the Dashboard is not showing on these stations. You may try to aim at them as well. Your mission is complete once you have successfully created errant sensor data. |
|  | Step 4. This completes your mission. Tap the Reset card on to the RFID reader so that the next participant can begin their mission. |

|  | Cyber-physical data must be protected from tampering and restricted to authorized access while in storage, transit and processing. |
| --- | --- |

|  | Research further the Raspberry PI version 2 “Xenon Death Flash”. Perform the follow-on laser lab to explore more about the photo-electric effect and the concept of Light Commands. |
| --- | --- |

RFID Tag

|  | CybatiWorks™ MISSION: Shapeshifting USB Devices |  |
| --- | --- | --- |
|  | This mission explores physical-cyber-attacks, physical devices and chips that can become malicious through software, firmware or hardware adjustments. USB devices have registered identifiers and represent themselves to the operating system as different devices. USB devices can have unique permissions based upon it’s OS identification that an attacker can take advantage of. In this mission you learn about Kautilya and its ability to program the Arduino platform device, the Teensyduino. |  |  |

|  | Step 1. Physically place the RFID tag located on this instruction sheet on top of the black RFID reader located to the left of the workstation. Remove the instruction card after you hear the BEEP. Click OK to start the Mission wizard. The wizard will prepare your Mission and then open a terminal prompt and the CybatiWorks Search Engine. First review the articles displayed in the CybatiWorks Search Engine. |
| --- | --- |
|  | Step 2. The wizard has automatically launched the Arduino sketch programming tool of Kautilya. Kautilya asks very simple questions to build its payload that can be transferred to the USB device called a Teensyduino. You can review the Kautilya options and prepare a payload. There largest variety of options is under Windows. |
|  | Step 3. Kautilya created an Arduino sketch that can be loaded on to a Teensyduino. The sketch programs the USB device to act like keyboard and mouse. The sketch can be adjusted to take advantage of other sensors or wireless transmitters soldered to the device. You have completed this mission once you created a sketch. It is beyond the scope of this example to load the firmware on to the Teensyduino and perform the exploit. Ask a facilitator for a Teensyduino so that you can see what it looks like. |
|  | Step 4. This completes your mission. Tap the Reset card on to the RFID reader so that the next participant can begin their mission. |

|  | Physical-cyber-attacks are very real and the attack can be much more sophisticated than just dropping a bunch of flash drives with file-based malware on them. Many chips and therefore devices use firmware to provide low-level functionality. These functions occur below common security controls such as Host-based IDS or anti-virus. |
| --- | --- |

|  | Check out the USB Rubber Ducky. The USB Rubber Ducky is a commercial version providing some of what the Teensyduino/Kautilya combination can perform. |
| --- | --- |

RFID Tag

|  | CybatiWorks™ MISSION: Simple Ladder Logic and Industrial Protocols |  |
| --- | --- | --- |
|  | This mission uses the AB PCCC protocol to communicate with the industrial Programmable Logic Controller (PLC). The logic influences a simple ladder logic program running on a MicroLogix 1400. You will learn to examine open / closed conditions and introductory Wireshark analysis of a protocol that does not have a complete Wireshark dissector (i.e. binary interpreter). |  |  |

|  | Step 1. Physically place the RFID tag located on this instruction sheet on top of the black RFID reader located to the left of the workstation. Remove the instruction card after you hear the BEEP. Click OK to start the Mission wizard. The wizard will prepare your Mission and then open the Chromium web browser. If the browser states the site is unavailable, wait patiently as the screen will refresh. |
| --- | --- |
|  | Step 2. The wizard launched a browser window with Node-RED logic, Dashboard, the conveyor.png ladder logic graphic and Wireshark. The Node-RED logic communicates using the AB PCCC protocol to the MicroLogix 1400 controller. The function files are located inside the Output and Input blocks within the PLC IP address configuration. The remote input B3:0/0 point influences the output in the first rung of the ladder logic displayed in the conveyor.png graphic. Examine if Open (i.e. if the bit location is a 0 then a true condition) and Examine if Closed (i.e. if the bit location is a 1 then a true condition) conditions are reviewed. Ask the facilitator to adjust the HDMI input to the Engineering Workstation. Toggle the 0 and 1 value for the RemoteInput to watch the rung transition between True and False causing the conveyer belt model to stop and start. |
|  | Step 3. The wizard also launched Wireshark with a display filter for AB PCCC’s TCP port number of 44818. The beginning of the capture collected the initial TCP 3-day handshake and the Common Industrial Protocol session negotiation. Inside of this data stream is the AB PCCC communication. Your mission is complete once you can catch only your 0 and 1 toggle bits. Try to create a Wireshark display filter to remove all of the normalcy, noisy traffic so that you can find only what matters, the anomaly of your 0 and 1 toggle bits. |
|  | Step 4. This completes your mission. Tap the Reset card on to the RFID reader so that the next participant can begin their mission. |

|  | Network analysis and interpretation can be very time consuming. It is very important to have a solid baseline of activity within your environment so the anomalous events stand out. It is also very important to know your cyber-physical logic so that you can understand the combined cyber-physical attack surface. |
| --- | --- |

|  | Make sure to also perform the Automation Exploit Wizard mission. |
| --- | --- |

RFID Tag

|  | CybatiWorks™ MISSION: Automation Exploit Wizard |  |
| --- | --- | --- |
|  | This mission loads a variety of exploit code and vulnerability notifications associated with automation technology. You will read the notification associated with CVE-2017-12088, review the exploit code and then execute the Python exploit against the programmable logic controller. |  |  |

|  | Step 1. Physically place the RFID tag located on this instruction sheet on top of the black RFID reader located to the left of the workstation. Remove the instruction card after you hear the BEEP. Click OK to start the Mission wizard. |
| --- | --- |
|  | Step 2. The wizard launched a window using a BASH scripting tool called Zenity. Select the default “Malformed Packet DoS using Python” exploit. This will launch the CybatiWorks search engine, a text editor with the exploit code and a command prompt within the exploit directory. You will then be prompted if you want to run the exploit now. Locate the MicroLogix 1400 controller. You may need to ask the facilitator to identify which controller you will impact. As you click OK watch the industrial controller operating at one of the stations. The exploit causes a fault and erases all logic within the device. |
|  | Step 3. After the controller has reset and is operating again, run the exploit again while running Wireshark to capture the interaction. You can also review the other CVEs; however, they are not fully automated. Let us know if you are interested in running one of the exploits against our mission controllers. Although these exploits are designed specifically for the MicroLogix 1400 controller, the attack categories exist for any type of industrial device. |
|  | Step 4. This completes your mission. Tap the Reset card on to the RFID reader so that the next participant can begin their mission. |

|  | Technology is composed of both software and hardware. Cyber-physical systems are composed of technology coupled with science to automate simple processes and the essentials of modern life. We must make sure to properly defend critical infrastructure. We can use your help to not allow these simple attacks to get in and furthermore defend against the complex attacks that can be life altering. |
| --- | --- |

|  | Exploits are not very hard to find and neither is performing the defense. Make sure you are committed to the process of learning about CVEs, applying the generality of the vulnerability and associating it with the Mitre att&ck framework. |
| --- | --- |

RFID Tag

|  | CybatiWorks™ MISSION: CybatiWorks Search Engine |  |
| --- | --- | --- |
|  | This mission explores the CybatiWorks Search Engine and the educational content inside of it. The search engine contains results from over a decade of work by some of the best analysts and defenders. The topics included in the search engine all relate to critical infrastructure and cyber-physical systems. |  |  |

|  | Step 1. Physically place the RFID tag located on this instruction sheet on top of the black RFID reader located to the left of the workstation. Remove the instruction card after you hear the BEEP. Click OK to start the Mission wizard. The wizard will prepare your Mission and then open the CybatiWorks Search Engine. |
| --- | --- |
|  | Step 2. The wizard has launched the integrated search engine with keywords of “CybatiWorks OR VxWorks”. Take some time to review the search results and perform some of your own queries. |
|  | Step 3. The CybatiWorks Search Engine is freely available in our full CybatiWorks VM. Contact us at info@cybati.com with a subject line of DefCon ICS Village and we will respond with the download location. |
|  | Step 4. This completes your mission. Tap the Reset card on to the RFID reader so that the next participant can begin their mission. |

|  | We highly recommend that everyone learn advanced google query language. It can speed up your open source intelligence research time dramatically and just make you happier overall. |
| --- | --- |

|  | Do you have something we should add to the Search Engine? Let us know of the resource location online and we will evaluate it. |
| --- | --- |

RFID Tag

|  | CybatiWorks™ MISSION: Remote Sensors Man-in-the-Middle |  |
| --- | --- | --- |
|  | This mission performs an Address Resolution Protocol (ARP) spoof to causing a selective protocol man-in-the-middle attack on the sensor ports TCP 50004 – 50006. The attack causes the Dashboard values to fluctuate to the given values in the Ettercap etterfilter. Man-in-the-middle attacks may occur many ways and this is an example exploiting the inherent, unauthenticated trust of ARP. |  |  |

|  | Step 1. Physically place the RFID tag located on this instruction sheet on top of the black RFID reader located to the left of the workstation. Remove the instruction card after you hear the BEEP. Click OK to start the Mission wizard. The wizard will prepare your Mission and then open the Chromium web browser. If the browser states the site is unavailable, wait patiently as the screen will refresh. |
| --- | --- |
|  | Step 2. The wizards launched the logic, Dashboard and a view at the hacker tools used by the P2 central station. Click the MitM enable button to instruct the P2 station to launch the attack. P2 is listening with a socat script to automatically launch the attack for 20 seconds. As you run the attack watch the ARP table window, the MAC address of your sensing station will change to the MAC address of the P2 station. After the attack is stopped the MAC address will be changed back to the correct value. |
|  | Step 3. Next, start Wireshark on eth0 and launch the attack again to collect the ARP spoof packets that cause the value to change. Try to create a filter to only catch the ARP traffic to make it easy to see the attack occur. This mission is complete once the filter is correct. |
|  | Step 4. This completes your mission. Tap the Reset card on to the RFID reader so that the next participant can begin their mission. |

|  | Man-in-the-middle attacks can occur for Domain Name Services (DNS), Network Time Protocol (NTP), Open Shortest Path First (OSPF), inter-process communications and lead to a fully compromised system. It is up to us, as defenders, to know what is normal, understand our processes and data flows and intentionally identify who and what we trust and for how long. |
| --- | --- |

|  | You can also review other attack tools like DeLorean for NTP or yersinia for infrastructure protocols. Think about how to defend against attacks by validation of integrity, strong authentication, restricted access, and the two-person rule. Everything must be balanced with the threat risk and value of the asset to modern life. |
| --- | --- |

RFID Tag

|  | CybatiWorks™ MISSION: Make Your Own LED Matrix Design |  |
| --- | --- | --- |
|  | This mission is to make a static or animated design for the LED matrix using Node-RED. |  |  |

|  | Step 1. Physically place the RFID tag located on this instruction sheet on top of the black RFID reader located to the left of the workstation. Remove the instruction card after you hear the BEEP. Click OK to start the Mission wizard. The wizard will prepare your Mission and then open the Chromium web browser. If the browser states the site is unavailable, wait patiently as the screen will refresh. |
| --- | --- |
|  | Step 2. The wizard has loaded the logic for the smiling and frowning faces used in the other missions. Your mission is to alter the block logic to create a new design. The new design can use any of the blocks within the palette on the left. Below is the document we used to create the smiling face. The final payload for Node-RED is at the bottom of the document after interpreting the matrix x and y coordinates. After building your model in the document, you will need to transfer your interpreted logic to Node-RED. |
|  | Step 3. Let us know once you have completed a new design. Will give you named or alias credit for it. We will save it in our mission set for future participants! |
|  | Step 4. This completes your mission. Tap the Reset card on to the RFID reader so that the next participant can begin their mission. |

|  | LED displays are located around the world for different purposes. Some are purely for advertising, while some are for alerts and notifications. Programming of these systems is often performed wirelessly and may not be properly secured. We have heard routinely in the news about Zombie apocalypse road signs. These displays should be properly protected with tools as simple as OpenVPN with public and private keys. |
| --- | --- |

|  | Search online for results of compromised road signs and review how OpenVPN, a properly configured crypto-system and physical security controls can be used to protect them. |
| --- | --- |

RFID Tag

|  | CybatiWorks™ MISSION: Maritime Automatic Identification System (AIS) |  |
| --- | --- | --- |
|  | This mission is to make a static or animated design for the LED matrix using Node-RED. |  |  |

|  | Step 1. Physically place the RFID tag located on this instruction sheet on top of the black RFID reader located to the left of the workstation. Remove the instruction card after you hear the BEEP. Click OK to start the Mission wizard. The wizard will prepare your Mission and then open the Chromium web browser. If the browser states the site is unavailable, wait patiently as the screen will refresh. |
| --- | --- |
|  | Step 2. The wizard has |
|  | Step 3. Let us know |
|  | Step 4. This completes your mission. Tap the Reset card on to the RFID reader so that the next participant can begin their mission. |

|  | LED displays |
| --- | --- |

|  | Search |
| --- | --- |

RFID Tag

|  | CybatiWorks™ MISSION: Software Defined Radios |  |
| --- | --- | --- |
|  | This mission is to make |  |  |

|  | Step 1. Physically place the RFID tag located on this instruction sheet on top of the black RFID reader located to the left of the workstation. Remove the instruction card after you hear the BEEP. Click OK to start the Mission wizard. The wizard will prepare your Mission and then open the Chromium web browser. If the browser states the site is unavailable, wait patiently as the screen will refresh. |
| --- | --- |
|  | Step 2. The wizard has loaded |
|  | Step 3. Let us know |
|  | Step 4. This completes your mission. Tap the Reset card on to the RFID reader so that the next participant can begin their mission. |

|  | LED displays |
| --- | --- |

|  | Search online |
| --- | --- |